SRT Ver.1.4 程序使用说明

连接说明：首先确保编程卡与电池连接并通过Type-C数据线连接PC端，在PC端下载并安装好SRT-USB-Link V1.4软件后，打开软件界面会自动识别连接端口，如下图所示的COM4，若后续操作过程中出现问题，有可能是因为默认识别的端口是错误的，手动切换到其他COM端口尝试即可。

SRT Ver.1.4 Program Usage Instructions

Connection Instructions: first, make sure the program box is connected to a battery and linked to your PC via the Type‑C data cable. Then download and install the SRT‑USB‑Link V1.4 software on your PC. Once the software is launched, it will automatically detect the connection port, just like the COM4 as shown in the figure below. If you encounter any issues during subsequent operations, it may be caused by the default detected port is incorrect. In that case, please switch to another COM port manually and try again.

Figure 1 Settings Interface

- 调参界面

- MP	—— Max Power

- BT	—— Boost

- DB	—— Dead Band

- T	—— Tension

- F	—— Force

- B	—— Brake

- Center  L0—0—R0

- Soft Start   Y

- Write Data

- Read Data

- Default

界面功能: Open / Save可打开或保存预设参数文件，

Read Data:	读取舵机参数

Write Data:	写入舵机参数

Default:		还原舵机出厂参数

1.Settings Interface

- MP	—— Max Power

- BT	—— Boost

- DB	—— Dead Band

- T	—— Tension

- F	—— Force

- B	—— Brake

- Center  L0—0—R0

- Soft Start   Y

- Write Data

- Read Data

- Default

Function:

Open / Save:	open or save the preset parameters file

Read Data:		read servo parameters

Write Data:		write servo parameters

Default:			restore servo parameters to the factory setting

Figure 2 Servo Test

- 宽/窄频PWM调角测度

Broad Band PWM  500us  900us  1500us  2100us  2500us

Narrow Band PWM 500us  750us  1000us

2. Servo Test

Broad Band PWM  500us  900us  1500us  2100us  2500us

Narrow Band PWM 500us  750us  1000us

Figure 3 Information

- 编程卡程序升级

点击Information选项，然后点击Load firmware 打开升级程序文件 “Program Upgrade Package Ver.1.4 —— SRT.bin ”, 再点击LCD Upgrade  执行升级操作（显示更新100%表示完成升级）

3.Information

Click the “Information” option, then click “Load firmware” to open the upgrade program file “Program Upgrade Package Ver.1.4 – SRT.bin”. After that, click “LCD Upgrade” to perform the upgrade. (When the progress shows 100% updated, the upgrade is complete.)

4.遇到问题的解决办法

1.保证编程卡与电池连接；

2.使用原配的TYPE-C线连接；

3.若电脑上有其他的USB设备连接，可能会导致软件自动识别的COM口错误，需要手动选择其他COM口直至成功；

4.如果遇到中途断电或其他原因导致屏幕黑屏，重新操作步骤即可。

注：早期编程卡版本，升级时需要：

1.下载最新的Ver.1.4编程软件

2.出现无法升级提示时，需要先连接原可识别的舵机，编程卡解锁界面后才能升级操作

4.Solutions for problems encountered

1.Make sure the programming card is properly connected to a battery.

2.Use the original Type‑C cable that came with the unit.

3.If other USB devices are connected to your PC, the software may automatically detect the wrong COM port. In that case, manually try different COM ports until the connection succeeds.

4.If the screen goes black due to a power interruption or other reasons during the process, simply repeat the steps from the beginning will be ok.

Note: For the previous program box version, upgrade requires:

1.Download the latest Ver.1.4 programming software.

2.If the upgrade fails, you need to connect the original recognizable servo first, and the program box can be upgraded only after unlocking the interface.